HANOI UNIVERSITY OF SCIENCE AND TECHNOLOGY SCHOOL OF ELECTRONIC AND TELECOMMUNICATION LSI CONTEST REGIONAL MAXIMA Team name: EDABK – HUST Instructor: Assoc. Prof. Nguyen Duc Minh Team members: Pham Quang Anh; K63; Hanoi University of Science and Technology. Nguyen Viet Thi; K63; Hanoi University of Science and Technology. Nguyen Duc Quang; K63; Hanoi University of Science and Technology. Hanoi, 01-2023

INFORMATION PAGE

- Team name: EDABK – HUST

- Task level of design: Level 3 (Advanced) - Improved algorithm, unlimited image size can be processed

- Members’ information:

- Pham Quang Anh

  - School’s name: Hanoi University of Science and Technology

  - Grade: 63

  - Address: No. 1 Dai Co Viet street, Hai Ba Trung, Hanoi, Vietnam

  - Mail address: anh.pq182359@sis.hust.edu.vn

  - Size of T-shirt: M

- Nguyen Viet Thi

  - School’s name: Hanoi University of Science and Technology

  - Grade: 63

  - Address: No. 1 Dai Co Viet street, Hai Ba Trung, Hanoi, Vietnam

  - Mail address: thi.nv182798@sis.hust.edu.vn

  - Size of T-shirt: XL

- Nguyen Duc Quang

  - School’s name: Hanoi University of Science and Technology

  - Grade: 63

  - Address: No. 1 Dai Co Viet street, Hai Ba Trung, Hanoi, Vietnam

  - Mail address: quang.nd182736@sis.hust.edu.vn

  - Size of T-shirt: XL

TABLE OF CONTENTS

TABLE OF FIGURES	i

TABLE OF TABLES	iii

CHAPTER 1.	NEW PROPOSED ALGORITHM	5

1.1 Restrictions of original algorithm	5

1.2 New proposed algorithm	6

1.2.1 Main idea	6

1.2.2 Flow chart	8

1.2.3 Example	9

1.2.4 Operation selecting the next element to be considered	13

1.2.5 Impovements and trade-offs of new algorithm	14

CHAPTER 2.	BLOCK DESIGN	15

2.1 Top module	15

2.1.1 Port description	15

2.1.2 Functional description	16

2.1.3 Architecture	17

2.1.4 Timing waveforms	18

2.2 Sub module	18

2.2.1 eda_controller	18

2.2.2 eda_compare	21

2.2.3 eda_fifos	23

2.2.4 eda_img_ram	24

2.2.5 eda_iterated_ram	26

2.2.6 eda_strobe_ram	29

2.2.7 eda_output_ram	31

CHAPTER 3.	RESULT	33

3.1 Simulation result	33

3.1.1 Software	33

3.1.2 Hardware	33

3.2 Synthesis results	35

CHAPTER 4.	HDL CODE	37

4.1 eda_regional_max	37

4.2 eda_controller	40

4.3 eda_compare	44

4.4 eda_fifos	46

4.5 eda_img_ram	53

4.6 eda_iterated_ram	55

4.7 eda_strobe_ram	58

4.8 eda_output_ram	60

4.9 eda_max	61

4.10 eda_one_hot_to_bin	62

4.11 sync_fifo	62

- TABLE OF FIGURES

Figure 1.1 Matlab result in case of large number of loops	5

Figure 1.2 The simplest case of finding regional maxima	6

Figure 1.3 The case when the area to be considered is expanded	7

Figure 1.4 Proposed algorithm flow  chart	8

Figure 1.5 Process 1 of the new algorithm	9

Figure 1.6 Iterate element in position row 1, column 1	10

Figure 1.7 The first step of expanding selected area	10

Figure 1.8 The last step of expanding selected area	11

Figure 1.9 Process update output	11

Figure 1.10 Status of four matrices after the first iteration	12

Figure 1.11 The next iteration	12

Figure 1.12 The result of the new algorithm	13

Figure 1.13 Example of operation selecting the next element to be considered	14

Figure 2.1 eda_regional_maxima block diagram	15

Figure 2.2 eda_regional_maxima architecture	17

Figure 2.3 eda_regional_maxima timing waveform	18

Figure 2.4 eda_controller block diagram	18

Figure 2.5 eda_controller finite state machine	21

Figure 2.6 eda_compare block diagram	21

Figure 2.7 eda_fifos block diagram	23

Figure 2.8 eda_img_ram block diagram	24

Figure 2.9 eda_iterated_ram block diagram	26

Figure 2.10 eda_strobe_ram block diagram	29

Figure 2.11 eda_output_ram block diagram	31

Figure 3.1 Load image into internal memory	33

Figure 3.2 Waveform of 1 testcase	34

Figure 3.3 Waveform of multiple testcases	34

Figure 3.4 Result compare with model	35

- TABLE OF TABLES

Table 2.1 eda_regional_maxima port description	15

Table 2.2 eda_regional_maxima parameter description	16

Table 2.3 eda_controller port description	19

Table 2.4 eda_controller parameter description	20

Table 2.5 eda_controller internal signals description	20

Table 2.6 eda_compare port description	22

Table 2.7 eda_compare parameter description	22

Table 2.8 eda_fifos port description	23

Table 2.9 eda_fifos parameter description	24

Table 2.10 eda_img_ram port description	25

Table 2.11 eda_img_ram parameter description	25

Table 2.12 eda_iterated_ram port description	26

Table 2.13 eda_iterated_ram parameter description	28

Table 2.14 eda_strobe_ram port description	29

Table 2.15 eda_strobe_ram parameter description	30

Table 2.16 eda_output_ram port description	31

Table 2.17 eda_output_ram parameter description	32

Table 3.1 Synthesis results	35

Table 4.1 eda_regional_max code	37

Table 4.2 eda_controller code	40

Table 4.3 eda_compare code	44

Table 4.4 eda_fifos code	46

Table 4.5 eda_img_ram code	53

Table 4.6 eda_iterated_ram code	55

Table 4.7 eda_strobe_ram code	58

Table 4.8 eda_output_ram code	60

Table 4.9 eda_max code	61

Table 4.10 eda_one_hot_to_bin code	62

Table 4.11 sync_fifo code	62

# NEW PROPOSED ALGORITHM

This chapter will show the restrictions of Regional Maxima algorithm was mentioned in design specification and propose a new algorithm which can remove those restrictions but still give correct result.

## Restrictions of original algorithm

With the original algorithm, the number of iterations needed to get the final result is unknown, because we don’t know when output is equal to output_cp. In the case there are many elements in a region have the same value, checking when output is equal to output_cp will increase the number of iterations significantly.

Figure 1.1 Matlab result in case of large number of loops

Figure 1.1 is the result in Matlab of example of the case there are many elements in a region of  matrix have the same value. As we can see, the algorithm need to run 5 loops until the end. Each loop contains 36 iterations over 36 elements of the matrix. In total, 180 iterations are required for this case. For hardware, if each element of matrix needs one cycle to traverse, the algorithm spend 180 cycles running to get the final result.

The best case is when all elements of the matrix have the same value, number of iterations in this case is equal to number of elements. Thus, except for the best case, the number of iterations (or cycles for hardware) is always greater than the number of elements of the matrix.

## New proposed algorithm

In this section, we will propose an algorithm which can remove above restrictions. The biggest goal of modifying algorithm is to shorten execution time by reducing number of iterations. Or in hardware’s parlance, this means reduce the number of cycles to get the final result. In the proofs below, we will show that this number of iterations is always fixed and equal to the number of image elements for all cases of different image’s contents. Of course, this algorithm must ensure output matrix will be correct by reasoning and simulating at the software level.

### Main idea

Detailed goal of our new algorithm is to replace or remove checking when output is equal to output_cp, because as analyzed above, this operation will cause number of iterations to increase.

In the simplest case when we consider an element of the matrix, basic principle of original algorithm is to find regional maxima in an area whose size is equal to window’s size (3x3) as described in Figure 1.2. In this case, the task of the algorithm is to check if the element colored in red in the matrix is greater than all the elements in yellow.

Figure 1.2 The simplest case of finding regional maxima

But Figure 1.2 is just the simplest possible case. If we are looking at the element "2" at the position of (row 1, column 5), we will see that in the 3x3 area around this element, there is an element "2" at the position of (row 2, column 4). Continue, we will see around the element "2" in (row 2, column 4), there is an element "2" at the position of (row 3, column 4) is in its 3x3 area. At this time, the task of the algorithm is to check if the value of all red elements is greater than the yellow elements surrounding them, not just check for a single red element.

Figure 1.3 The case when the area to be considered is expanded

Figure 1.3 explains why the original algorithm needs to check the condition 2 "(the value of pixel A in any 8-neighborhood) = (the value of the center pixel)" and the stopping condition of the algorithm is “output == output_cp”. This means, the purpose of these two conditions is to ensure that all equal neighboring elements in the input image will generate the same value for the corresponding elements in the output matrix. In the case of Figure 1.3, elements at positions (rows 1, column 5), (row 2, column 4), (row 3, column 4) of matrix output will have value 0 because the value of red elements in Figure 1.3 is not geater than their sorrounding yellow elements.

Applying the analysis above, instead of checking when the output is equal to output_cp, we will expand considered area as shown in Figure 1.3. The corresponding values on the output matrix of the considered elements (red elements as shown in the Figure 1.3) will be updated at the same time. This ensures that all red elements will have the same value without using condition 2 "(the value of pixel A in any 8-neighborhood) = (the value of the center pixel)" and stopping condition “output == output_cp”.

A condition for this new algorithm is which elements have already been checked will not be checked again. For example, if the element in (row 3, column 4) of the output matrix has been updated, then when browsing to (row 3, column 3), we will not go to (row 3, column 4), but switch immediately to browse the position of (row 3, column 5).

The detailed operations of the algorithm will be presented in sections 1.2.2 and 1.2.3.

### Flow chart

Figure 1.4 Proposed algorithm flow  chart

Note:

- i , j:  Index of element

- ones: assign all element of matrix to 1

- zeros: assign all element of matrix to 0

- iterated_idx: The matrix that mark the elements of input image are iterated or not, each element of  iterated_idx matrix corresponds to  an element of input image.

- out_compare: Temperal compare result of current iteration. After the end of current iteration, all selected elements are assigned out_compare.

- strobe: The matrix that mark the elements of input image are selected in current iteration or not, each element of  strobe matrix corresponds to  an element of input image.

### Example

- Process 1: Prepare matrix input to store the input image, matrix output initialized with true (1), matrix strobe to mark the selected elements in the current iteration, initialized with false (0), matrix iterated_idx to mark the iterated elements, initialized with fale (0) as shown in Figure 1.5.

Figure 1.5 Process 1 of the new algorithm

- Process 2: Start with element at position (row 1, column 1). The red element of matrix input in Figure 1.6 is the selected element in the current iteration. The yellow elements of matrix input in Figure 1.6 are the neighbors of selected element. At the time when element (row 1, column 1) of input is selected, position (row 1, column 1) of matrix strobe will be updated to true (1) as shown in Figure 1.6. Also, the element at position (row 1, column 1) of matrix iterated_idx will be updated to true (1) too. This means element at position (row 1, column 1) is iterated, and will never be traversed or re-selected in any future iterations.

Figure 1.6 Iterate element in position row 1, column 1

The task now is to check if any of the neighbors are equal to the red element. We will see the elements at position (row 1, column 2) and (row 2, column 1) are equal to red element. So, the elements (row 1, column 2) and (row 2, column 1) now are the selected elements too. Neighboring elements now are all the elements colored yellow in matrix input as shown in Figure 1.7 because we add neighboring elements within 3x3 area of 2 new red elements. This process is called expanding area process.

Figure 1.7 The first step of expanding selected area

Once again, we see some neighbors of elements (1, 2) and (2, 1) have value “0” too (positions (1, 3), (2, 3), (3, 1), (3, 2)). The selected area will continue to expand, the selected elements will continue to be added. And this process only stops when there are no more neighbors of the matrix input’s red elements have value “0”. Matrices input, strobe, iterated_idx now are updated as shown in Figure 1.8.

Figure 1.8 The last step of expanding selected area

In case no need to expand the selected area, just go to process 3.

- Process 3: After expanding the selected area is finished, the next task is to check if the red elements in matrix input have greater value than all the yellow elements. In this case, it’s false. We will see in the matrix strobe which positions have a value of “1”, then we will update value “0” for all elements with the corresponding positions on the matrix output. The value of matrix output now is shown as Figure 1.9.

Figure 1.9 Process update output

- Process 4: The last iteration is finished. Matrix strobe now is cleared. Current status of four matrices are shown as Figure 1.10.

Figure 1.10 Status of four matrices after the first iteration

In this process, we need to check if all elements of matrix iterated_idx is “1”. If not, the algorithm have not done yet, and we will go to the next iteration.

Note: By applying operation selecting the next element to be considered in 1.2.4, the algorithm will ignore elements whose corresponding values in matrix iterated_idx is “1”. Example, in this case, elements (row 1, column 2), (row 1, column 3) have corresponding values in matrix iterated_idx is “1”. So, we will skip them. The next iteration is to consider element (row 1, column 4).

From now on, process 2 and 3 will be repeated. So, after process 3, status of four matrices now are updated as shown in Figure 1.11.

Figure 1.11 The next iteration

If all elements of matrix iterated_idx are “1”, the algorithm is finished. In other words, the stopping condition of the algorithm is that all elements of iterated_idx are equal to “1”. We can say that, after traversing all elements of  matrix input, all elements of matrix output have been updated correctly. The result is shown as Figure 1.12.

Figure 1.12 The result of the new algorithm

### Operation selecting the next element to be considered

If we iterate through each element of matrix input in turn without skipping the elements which have the corresponding iterated_idx equal to “1”, redundant iterations will appear. For example, consider the example in 1.2.3, after the first iteration, elements (1, 1), (1, 2) are obviously already selected from the previous iteration, but if we keep doing in turn without skipping them, this will cause these 2 elements to be selected again redundantly. This is also a restriction of the original algorithm if you look at the Matlab code.

We propose an operation selecting the next element to be considered to solve this problem. The idea is like this:

- Through the matrix iterated_idx, we know which elements have been selected before. So our job is simply to find the row position of iterated_idx where at least one element is equal to "0" and on that row, find the position of the first zero element from left to right.

- The position of the row and column just found is the position of the next selected element.

For example, if we know the state of iterated_idx is like Figure 1.13, we will see the row position of iterated_idx where at least one element is equal to "0" is row 3 and the position of the first zero element from left to right on row 3 is column 4. So, the next selected element’s position is (3, 4).

Figure 1.13 Example of operation selecting the next element to be considered

### Impovements and trade-offs of new algorithm

#### Improvements

- As we can see in section 1.2.3, number of iterations is always equal to number of input’s elements regardless of input’s content. So, except for the best case where all the elements of input have the same value, the number of iterations reduces. In the case of Figure 1.1 (worst case), the number of iterations reduces 5 times (from 180 to 36). In the case of section 1.2.3, the number of iterations reduces 2 times (from 72 to 36). It also means that the number of execution cycles of the hardware significantly reduces.

- We support the operation selecting the next element to be considered, the purpose is to skip the selected elements in the previous iterations, thereby, helping to eliminate redundant iterations, making efficiency optimal. Looking at the old algorithm's Matlab code, we can see that in loop 2, each element of matrix input have one iteration. This creates redundant iterations, making efficiency suboptimal.

#### Trade-offs

- As mentioned above, new algorithm needs more hardware resources than the original one to serve operation expanding the selected area and to store iterated_idx to serve the operation calculating to ignore the previously selected elements. However, since each element of iterated_idx is only 1 bit , the circuit size for implementing the new algorithm is not too large compared to the old algorithm.

# BLOCK DESIGN

This chapter describes the overall and detailed design of the hardware that implements our Regional Maxima algorithm in CHAPTER 1.

## Top module

Figure 2.1 eda_regional_maxima block diagram

### Port description

Table 2.1 eda_regional_maxima port description

| Signal name | Width | I/O | Description |
| --- | --- | --- | --- |
| clk | 1 | Input | Clock signal |
| reset_n | 1 | Input | Asynchronous reset, active LOW |
| write_en | 1 | Input | Write enable |
| wr_addr | ADDR_WIDTH | Input | Write address {i, j} |
| pixel_in | PIXEL_WIDTH | Input | Pixels’ value user want to write to internal image memory of top module in the current cycle |
| start | 1 | Input | Start running algorithm after loading image, active HIGH |
| done | 1 | Input | Notice that algorithm has been done |
| matrix_output | M*N | Input | Matrix output – result of the algorithm |

Table 2.2 eda_regional_maxima parameter description

| Parameter | Value | Description |
| --- | --- | --- |
| M | 2 to 256 Default: 6 | Height of image |
| N | 2 to 256 Default: 6 | Width of image |
| ADDR_WIDTH | $clog2(M) + $clog2(N) | Number of address bits to store image |
| PIXEL_WIDTH | 8 or 16 Default: 8 | Number of bits to store a pixel’s value |

### Functional description

eda_regional_maxima is the hardware that implements Regional Maxima algorithm in CHAPTER 1 with an unlimited size and pixel storage bits image, fixed filter window size (3x3). To suit actual use, we support image height from 2 to 256 (pixels), image width from 2 to 256 (pixels) and pixel size of 8 or 16 bits.

Before running algorithm, it is necessary to run the image loading process. This process helps us to store the pixel values taken from signal pixel_in to address wr_addr of  top module’s internal image memory. Note that in one cycle, only one pixel (8 bits or 16 bits) is loaded to the image memory. Such design is intended to match the actual RAM where in each read/write command the number of data bits read/write is only 1/4/8/16/32 bits.

After image has already been loaded, user can activate signal start to tell the eda_regional_maxima to start running the algorithm. Note that if image has not been completely loaded yet, activate signal start still cause eda_regional_maxima start running algorithm. So, it will cause fault result. Make sure start is active only when image has already been loaded.

After exactly M*N cycles when start is active, done is HIGH. This means the algorithm has been done and the values of matrix_output[i][j] (i, j are positions of pixel) is now the true result. Note that if done is LOW (algorithm has not been done), matrix_output still has value but that is not meaningful (not valid). Once the result of the old image is valid, user can continue to load new images and run Regional Maxima algorithm with new images.

### Architecture

Figure 2.2 is eda_regional_maxima’s architecture.

Figure 2.2 eda_regional_maxima architecture

eda_regional_maxima is designed with the following sub-modules:

- eda_controller is an FSM used to control all blocks by the following opertations:

  - Write new value to all RAMs except for eda_img_ram.

  - Clear values in all RAMs except for eda_output_ram.

  - Signal to other modules when to start and finish the execution of the algorithm.

  - Expand or stop expanding selected area then switch to the next iteration.

  - Calculate the next selected address (center_addr).

  - Determine when to read and when to stop reading FIFOs.

- eda_compare is responsible for checking if the selected elements are greater than all the neighboring elements of them and checking if any neighboring elements are equal to the selected elements.

- eda_fifos contains 8 fifos, each fifo stores the position of one neighboring pixel relative to the currently selected pixel: up left, up, up right, left, right, down left, down, down right if they have the same value as the selected pixel.

- eda_img_ram is the internal memory which is used to store input image’s content and provide the neighboring addresses of the selected image, determining which positions on the 3x3 window are the valid positions (not out of matrix input).

- eda_iterated_ram is the internal memory which is used to store matrix iterated_idx as decribed in CHAPTER 1 and perform the operation selecting the next element to be considered in CHAPTER 1.

- eda_strobe_ram is the internal memory which is used to store matrix strobe as decribed in CHAPTER 1 provide and provide the currently selected positions to other modules.

- eda_output_ram updates the matrix output based on strobed signal.

### Timing waveforms

Figure 2.3 is the timing waveform of eda_regional_maxima.

Figure 2.3 eda_regional_maxima timing waveform

## Sub module

### eda_controller

Figure 2.4 eda_controller block diagram

#### Port description

Table 2.3 eda_controller port description

| Signal name | Width | I/O | Description |
| --- | --- | --- | --- |
| clk | 1 | Input | Clock signal |
| reset_n | 1 | Input | Asynchronous reset, active LOW |
| start | 1 | Input | Start running algorithm after loading image, active HIGH |
| iterated_all | 1 | Input | Notice that all pixels are iterated |
| fifo_empty | WINDOW_WIDTH - 1 | Input | FIFOs empty flags |
| push_positions | WINDOW_WIDTH - 1 | Input | Positions need to be pushed into FIFOs |
| data_out | ADDR_WIDTH | Input | Address retrieved from FIFOs |
| next_row | I_WIDTH | Input | If selected area is not needed to expand, this is the next row of next iteration |
| next_col | J_WIDTH | Input | If selected area is not needed to expand, this is the next column of next iteration |
| new_pixel | 1 | Output | Flag signals that there is a new pixel is selected if the algorithm has not been done |
| read_en | WINDOW_WIDTH | Output | FIFOs read enable |
| clear | 1 | Output | Clear RAMs (eda_img_ram, eda_iterated_ram and eda_strobe_ram) |
| center_addr | ADDR_WIDTH | Output | The next selected address |
| pre_center_addr | ADDR_WIDTH | Output | Predicted value of the next selected address (same value but 1 cycle earlier than center_addr) |
| update_strb | 1 | Output | Flag update strobe |
| done | 1 | Output | Flag done (algorithm has been done) |

Table 2.4 eda_controller parameter description

| Parameter | Value | Description |
| --- | --- | --- |
| WINDOW_WIDTH | 9 | Number of pixels in the area of a filter window |
| ADDR_WIDTH | $clog2(M) + $clog2(N) | Number of address bits to store image |
| I_WIDTH | $clog2(M) | Number of bits to store input's rows |
| J_WIDTH | $clog2(N) | Number of bits to store input's columns |

#### Functional description

eda_controller is an FSM used to control all blocks by the following opertations:

- Write new value to all RAMs except for eda_img_ram.

- Clear values in all RAMs except for eda_output_ram.

- Signal to other modules when to start and finish the execution of the algorithm.

- Expand or stop expanding selected area then switch to the next iteration.

- Calculate the next selected address (center_addr).

- Determine when to read and when to stop reading FIFOs.

#### Finite State Machine

Figure 2.5 is the FSM of eda_controller with some default assignments and reset values described in Table 2.5.

Table 2.5 eda_controller internal signals description

| Signal | Mode | Type | Scheme | Default | Reset |
| --- | --- | --- | --- | --- | --- |
| clear | output | reg | Combinational | 0 |  |
| update_strb | output | reg | Combinational | 0 |  |
| new_pixel | output | reg | Clocked |  | 0 |
| done | output | reg | Clocked |  | 0 |
| update_addr | local | reg | Combinational | 0 |  |
| extend_addr | local | reg | Combinational | 0 |  |
| check_next | local | wire | Combinational | 0 |  |

Figure 2.5 eda_controller finite state machine

### eda_compare

Figure 2.6 eda_compare block diagram

#### Port description

Table 2.6 eda_compare port description

| Signal name | Width | I/O | Description |
| --- | --- | --- | --- |
| clk | 1 | Input | Clock signal |
| reset_n | 1 | Input | Asynchronous reset, active LOW |
| new_pixel | 1 | Input | Flag signals that there is a new pixel is selected if the algorithm has not been done |
| window_values | PIXEL_WIDTH * WINDOW_WIDTH | Input | Concatenation values of all pixels within the 3x3 window area |
| neigh_addr_valid | WINDOW_WIDTH - 1 | Input | Indicates which positions on the window are the valid positions (not out of matrix input) |
| iterated_idx | WINDOW_WIDTH - 1 | Input | The positions on the 3x3 window have already been selected before |
| compare_out | 1 | Output | Compare result |
| push_positions | WINDOW_WIDTH | Output | Positions need to be pushed into FIFOs |

Table 2.7 eda_compare parameter description

| Parameter | Value | Description |
| --- | --- | --- |
| WINDOW_WIDTH | 9 | Number of pixels in the area of a filter window |
| PIXEL_WIDTH | 8 or 16 Default: 8 | Number of bits to store a pixel’s value |

#### Functional description

eda_compare is responsible for checking if the selected elements are greater than all the neighboring elements of them. If true, compare_out is HIGH. If not, compare_out is LOW and eda_compare has to check if any neighboring elements are equal to the selected elements. If there are, which are they? The result will be shown on push_positions. If push_positions[i] is HIGH, this means neighboring element at position i is equal to the selected element.

### eda_fifos

Figure 2.7 eda_fifos block diagram

#### Port description

Table 2.8 eda_fifos port description

| Signal name | Width | Input/Output | Description |
| --- | --- | --- | --- |
| clk | 1 | Input | Clock signal |
| reset_n | 1 | Input | Asynchronous reset, active LOW |
| push_positions | WINDOW_WIDTH – 1 | Input | Positions need to be pushed into FIFOs |
| read_en | WINDOW_WIDTH – 1 | Input | FIFOs read enable signal |
| upleft_addr | ADDR_WIDTH | Input | Up left address |
| up_addr | ADDR_WIDTH | Input | Up address |
| upright_addr | ADDR_WIDTH | Input | Up right address |
| left_addr | ADDR_WIDTH | Input | Left address |
| right_addr | ADDR_WIDTH | Input | Right address |
| downleft_addr | ADDR_WIDTH | Input | Down left address |
| down_addr | ADDR_WIDTH | Input | Down address |
| downright_addr | ADDR_WIDTH | Input | Down right address |
| fifo_empty | WINDOW_WIDTH – 1 | Output | FIFOs empty flags |
| data_out | ADDR_WIDTH – 1 | Output | Address retrieved from FIFO |

Table 2.9 eda_fifos parameter description

| Parameter | Value | Description |
| --- | --- | --- |
| WINDOW_WIDTH | 9 | Number of pixels in the area of a filter window |
| ADDR_WIDTH | $clog2(M) + $clog2(N) | Number of address bits to store image |
| I_WIDTH | $clog2(M) | Number of bits to store input's rows |
| J_WIDTH | $clog2(N) | Number of bits to store input's columns |

#### Functional description

eda_fifos contains 8 fifos, each fifo stores the position of one neighboring pixel relative to the currently selected pixel: up left, up, up right, left, right, down left, down, down right.

If the 8 neighboring pixels have the same value as the currently selected pixel and it hasn't been checked in previous rounds, it will be pushed into the fifo.

### eda_img_ram

Figure 2.8 eda_img_ram block diagram

#### Port description

Table 2.10 eda_img_ram port description

| Signal name | Width | I/O | Description |
| --- | --- | --- | --- |
| clk | 1 | Input | Clock signal |
| reset_n | 1 | Input | Asynchronous reset, active LOW |
| write_en | 1 | Input | Write enable |
| wr_addr | ADDR_WIDTH | Input | Write address |
| pixel_in | PIXEL_WIDTH | Input | Pixel value that user want to write to image RAM |
| center_addr | ADDR_WIDTH | Input | The next selected address |
| window_values | PIXEL_WIDTH * WINDOW_WIDTH | Output | Concatenation values of all pixels within the 3x3 window area |
| neigh_addr_valid | WINDOW_WIDTH - 1 | Output | Indicates which positions on the window are the valid positions (not out of matrix input) |
| upleft_addr | ADDR_WIDTH | Output | Up left address |
| up_addr | ADDR_WIDTH | Output | Up address |
| upright_addr | ADDR_WIDTH | Output | Up right address |
| left_addr | ADDR_WIDTH | Output | Left address |
| right_addr | ADDR_WIDTH | Output | Right address |
| downleft_addr | ADDR_WIDTH | Output | Down left address |
| down_addr | ADDR_WIDTH | Output | Down address |
| downright_addr | ADDR_WIDTH | Output | Down right address |

Table 2.11 eda_img_ram parameter description

| Parameter | Value | Description |
| --- | --- | --- |
| ADDR_WIDTH | $clog2(M) + $clog2(N) | Number of address bits to store image |
| WINDOW_WIDTH | 9 | Number of pixels in the area of a filter window |
| PIXEL_WIDTH | 8 or 16 Default: 8 | Number of bits to store a pixel’s value |

#### Functional description

eda_img_ram is the internal memory which is used to store input image’s content and provide the neighboring addresses of the selected image, determining which positions on the 3x3 window are the valid positions (not out of matrix input).

### eda_iterated_ram

Figure 2.9 eda_iterated_ram block diagram

#### Port description

Table 2.12 eda_iterated_ram port description

| Signal name | Width | I/O | Description |
| --- | --- | --- | --- |
| clk | 1 | Input | Clock signal |
| reset_n | 1 | Input | Asynchronous reset, active LOW |
| clear | 1 | Input | Clear RAMs (eda_img_ram, eda_iterated_ram and eda_strobe_ram) |
| new_pixel | 1 | Input | Flag signals that there is a new pixel is selected if the algorithm has not been done |
| done | 1 | Input | Notice that algorithm has been done |
| fifo_empty | WINDOW_WIDTH - 1 | Input | FIFOs empty flags |
| center_addr | ADDR_WIDTH | Input | The next selected address |
| upleft_addr | ADDR_WIDTH | Input | Up left address |
| up_addr | ADDR_WIDTH | Input | Up address |
| upright_addr | ADDR_WIDTH | Input | Up right address |
| left_addr | ADDR_WIDTH | Input | Left address |
| right_addr | ADDR_WIDTH | Input | Right address |
| downleft_addr | ADDR_WIDTH | Input | Down left address |
| down_addr | ADDR_WIDTH | Input | Down address |
| downright_addr | ADDR_WIDTH | Input | Down right address |
| neigh_addr_valid | WINDOW_WIDTH - 1 | Input | Indicates which positions on the window are the valid positions (not out of matrix input) |
| push_positions | WINDOW_WIDTH - 1 | Input | Positions need to be pushed into FIFOs |
| iterated_idx | WINDOW_WIDTH - 1 | Output | The positions on the 3x3 window have already been selected before |
| next_row | I_WIDTH | Output | If selected area is not needed to expand, this is the next row of next iteration |
| next_col | J_WIDTH | Output | If selected area is not needed to expand, this is the next column of next iteration |
| sel_row | M | Output | It’s next_row in the form of one hot code |
| sel_col | M * N | Output | Predict the next column that will be selected of all rows in the form of one hot code |
| iterated_all | 1 | Output | Notice that all pixels are iterated |

Table 2.13 eda_iterated_ram parameter description

| Parameter | Value | Description |
| --- | --- | --- |
| M | 2 to 256 Default: 6 | Height of image |
| N | 2 to 256 Default: 6 | Width of image |
| I_WIDTH | $clog2(M) | Number of bits to store input's rows |
| J_WIDTH | $clog2(N) | Number of bits to store input's columns |
| ADDR_WIDTH | $clog2(M) + $clog2(N) | Number of address bits to store image |
| WINDOW_WIDTH | 9 | Number of pixels in the area of a filter window |

#### Functional description

eda_iterated_ram is the internal memory which is used to store matrix iterated_idx as decribed in CHAPTER 1 and perform the operation selecting the next element to be considered in CHAPTER 1. The result of operation selecting the next element to be considered will be shown in next_row, next_col, sel_row, sel_col.

### eda_strobe_ram

Figure 2.10 eda_strobe_ram block diagram

#### Port description

Table 2.14 eda_strobe_ram port description

| Signal name | Width | I/O | Description |
| --- | --- | --- | --- |
| clk | 1 | Input | Clock signal |
| reset_n | 1 | Input | Asynchronous reset, active LOW |
| update_strb | 1 | Input |  |
| clear | 1 | Input | Clear RAMs (eda_img_ram, eda_iterated_ram and eda_strobe_ram) |
| new_pixel | 1 | Input | Flag signals that there is a new pixel is selected if the algorithm has not been done |
| iterated_all | 1 | Input | Notice that all pixels are iterated |
| done | 1 | Input | Notice that algorithm has been done |
| pre_center_addr | ADDR_WIDTH | Input | Predicted value of the next selected address (same value but 1 cycle earlier than center_addr) |
| upleft_addr | ADDR_WIDTH | Input | Up left address |
| up_addr | ADDR_WIDTH | Input | Up address |
| upright_addr | ADDR_WIDTH | Input | Up right address |
| left_addr | ADDR_WIDTH | Input | Left address |
| right_addr | ADDR_WIDTH | Input | Right address |
| downleft_addr | ADDR_WIDTH | Input | Down left address |
| down_addr | ADDR_WIDTH | Input | Down address |
| downright_addr | ADDR_WIDTH | Input | Down right address |
| sel_row | M | Input | It’s next_row in the form of one hot code |
| sel_col | M * N | Input | Predict the next column that will be selected of all rows in the form of one hot code |
| strb_value | M * N | Output | Pixels are being selected |

Table 2.15 eda_strobe_ram parameter description

| Parameter | Value | Description |
| --- | --- | --- |
| M | 2 to 256 Default: 6 | Height of image |
| N | 2 to 256 Default: 6 | Width of image |
| ADDR_WIDTH | $clog2(M) + $clog2(N) | Number of address bits to store image |

#### Functional description

eda_strobe_ram is the internal memory which is used to store matrix strobe as decribed in CHAPTER 1 provide and provide the currently selected positions to other modules.

### eda_output_ram

Figure 2.11 eda_output_ram block diagram

#### Port description

Table 2.16 eda_output_ram port description

| Signal name | Width | Input/Output | Description |
| --- | --- | --- | --- |
| clk | 1 | Input | Clock signal |
| reset_n | 1 | Input | Asynchronous reset, active LOW |
| clear | 1 | Input | Synchronous reset, active high, use when load new image |
| new_pixel | 1 | Input | Signal that indicate new pixel |
| update_strobe | 1 | Input | Signal that pixel updates are being selected |
| strb_value | M*N | Input | Pixels are being selected |
| matrix_output | M*N | Output | Matrix output |

Table 2.17 eda_output_ram parameter description

| Parameter | Value | Description |
| --- | --- | --- |
| M | 2 to 256 Default: 6 | Height of image |
| N | 2 to 256 Default: 6 | Width of image |

#### Functional description

eda_output_ram updates the matrix output based on strobed signal.

# RESULT

This chapter describes the simulation results and synthesis results. We use tools: Questasim,  Synopsys Design Compiler of Dolphin Technology Vietnam Center to simulate and synthesis.

## Simulation result

### Software

We code new proposed algorithm by Python (code is located in software/new_imregion_max.py) and compare with the results of the original algorithm (original algorithm is converted to Python code and is located in file software/imregion_max_orginal.py). After this, we simulate new algorithm with 1000000 test cases of image 16x16 (file software/test.py). So, the outputs of the new algorithm are equal to the original one.

The results of the software simulation mentioned above are located in folder software/test_result/.

### Hardware

We create 1000000 random images (size 10x10, each pixel value is between 0 and 5) and write a model with correct output to check the output of hardware code. Finally, the results are all correct.

The results is in the folder sim/tb/ and the log result is located in sim/tb/vsim.log.

Figure 3.1 Load image into internal memory

Figure 3.2 Waveform of 1 testcase

Figure 3.3 Waveform of multiple testcases

Figure 3.4 Result compared with model

## Synthesis results

We use 28nm library of Dolphin Technology Vietnam Center to synthesis and the maximum achievable frequency is 760 MHz.

The result of the synthesis phase is in the folder syn/reports/.

Table 3.1 Synthesis results

| Timing Path Group 'clk' | ----------------------------------- Levels of Logic: 16.00000 Critical Path Length: 0.41488 Critical Path Slack: 0.00005 Critical Path Clk Period: 1.31579 Total Negative Slack: 0.00000 No. of Violating Paths: 0.00000 Worst Hold Violation: -0.19615 Total Hold Violation: -65.60418 No. of Hold Violations: 646.00000 ----------------------------------- |
| --- | --- |
| Cell Count | ----------------------------------- Hierarchical Cell Count: 0 Hierarchical Port Count: 0 Leaf Cell Count: 5523 Buf/Inv Cell Count: 1001 Buf Cell Count: 40 Inv Cell Count: 961 CT Buf/Inv Cell Count: 1 Combinational Cell Count: 4754 Sequential Cell Count: 769 Macro Count: 0 ----------------------------------- |
| Area | ----------------------------------- Combinational Area: 2719.59799 Noncombinational Area: 1796.04602 Buf/Inv Area: 302.23200 Total Buffer Area: 49.00000 Total Inverter Area: 253.23200 Macro/Black Box Area: 0.00000 Net Area: 0.00000 ----------------------------------- Cell Area: 4515.64401 equivalent to 15359,3333 nand2x1 cells Design Area: 4515.64401 |
| Power | Global Operating Voltage = 0.81 V Total Dynamic Power = 2.08717 mW (100%) Cell Leakage Power = 313.1223 nW |

# HDL CODE

## eda_regional_max

Table 4.1 eda_regional_max code

`include "eda_global_define.svh" module eda_regional_max #( // synopsys template parameter M = `CFG_M, parameter N = `CFG_N, parameter ADDR_WIDTH = `CFG_ADDR_WIDTH, parameter WINDOW_WIDTH = `CFG_WINDOW_WIDTH, parameter PIXEL_WIDTH = `CFG_PIXEL_WIDTH, parameter I_WIDTH = `CFG_I_WIDTH, parameter J_WIDTH = `CFG_J_WIDTH ) ( // Port Declarations input wire clk, input wire [PIXEL_WIDTH- 1:0] pixel_in, input wire reset_n, input wire start, input wire [ADDR_WIDTH - 1:0] wr_addr, input wire write_en, output wire done, output wire [M - 1:0][N - 1:0] matrix_output ); // Internal Declarations // Local declarations // Internal signal declarations wire [ADDR_WIDTH - 1:0] center_addr; wire clear; wire update_strb; // Update strobe wire compare_out; wire [ADDR_WIDTH-1:0] data_out; // Data out from FIFO wire [ADDR_WIDTH - 1:0] down_addr; wire [ADDR_WIDTH - 1:0] downleft_addr; wire [ADDR_WIDTH - 1:0] downright_addr; wire [WINDOW_WIDTH - 2:0] fifo_empty; // FIFO empty wire iterated_all; wire [WINDOW_WIDTH - 2:0] iterated_idx; wire [ADDR_WIDTH - 1:0] left_addr; wire [WINDOW_WIDTH - 2:0] neigh_addr_valid; wire new_pixel; wire [J_WIDTH - 1:0] next_col; wire [I_WIDTH - 1:0] next_row; wire [WINDOW_WIDTH - 2:0] push_positions; wire [WINDOW_WIDTH - 2:0] read_en; wire [ADDR_WIDTH - 1:0] right_addr; wire [M - 1:0][N - 1:0] strb_value; wire [ADDR_WIDTH - 1:0] up_addr; wire [ADDR_WIDTH - 1:0] upleft_addr; wire [ADDR_WIDTH - 1:0] upright_addr; wire [PIXEL_WIDTH * WINDOW_WIDTH - 1:0] window_values; wire [ADDR_WIDTH-1:0] pre_center_addr; // Center address wire [M - 1:0] sel_row; wire [M - 1:0][N - 1:0] sel_col; // Instances eda_compare eda_compare( .clk (clk), .reset_n (reset_n), .new_pixel (new_pixel), .window_values (window_values), .neigh_addr_valid (neigh_addr_valid), .iterated_idx (iterated_idx), .compare_out (compare_out), .push_positions (push_positions) ); eda_controller eda_controller( .clk (clk), .reset_n (reset_n), .start (start), .iterated_all (iterated_all), .fifo_empty (fifo_empty), .push_positions (push_positions), .data_out (data_out), .next_row (next_row), .next_col (next_col), .new_pixel (new_pixel), .clear (clear), .center_addr (center_addr), .pre_center_addr (pre_center_addr), .update_strb (update_strb), .done (done), .read_en (read_en) ); eda_fifos eda_fifos( .clk (clk), .reset_n (reset_n), .read_en (read_en), .push_positions (push_positions), .upleft_addr (upleft_addr), .up_addr (up_addr), .upright_addr (upright_addr), .left_addr (left_addr), .right_addr (right_addr), .downleft_addr (downleft_addr), .down_addr (down_addr), .downright_addr (downright_addr), .fifo_empty (fifo_empty), .data_out (data_out) ); eda_img_ram eda_img_ram( .clk (clk), .reset_n (reset_n), .write_en (write_en), .wr_addr (wr_addr), .pixel_in (pixel_in), .center_addr (center_addr), .window_values (window_values), .neigh_addr_valid (neigh_addr_valid), .upleft_addr (upleft_addr), .up_addr (up_addr), .upright_addr (upright_addr), .left_addr (left_addr), .right_addr (right_addr), .downleft_addr (downleft_addr), .down_addr (down_addr), .downright_addr (downright_addr) ); eda_iterated_ram eda_iterated_ram( .clk (clk), .reset_n (reset_n), .clear (clear), .new_pixel (new_pixel), .done (done), .fifo_empty (fifo_empty), .center_addr (center_addr), .upleft_addr (upleft_addr), .up_addr (up_addr), .upright_addr (upright_addr), .left_addr (left_addr), .right_addr (right_addr), .downleft_addr (downleft_addr), .down_addr (down_addr), .downright_addr (downright_addr), .neigh_addr_valid (neigh_addr_valid), .push_positions (push_positions), .iterated_idx (iterated_idx), .next_row (next_row), .next_col (next_col), .sel_row (sel_row), .sel_col (sel_col), .iterated_all (iterated_all) ); eda_output_ram eda_output_ram( .clk (clk), .reset_n (reset_n), .clear (clear), .new_pixel (new_pixel), .update_strb (update_strb), .compare_out (compare_out), .strb_value (strb_value), .matrix_output (matrix_output) ); eda_strobe_ram eda_strobe_ram( .clk (clk), .reset_n (reset_n), .update_strb (update_strb), .clear (clear), .new_pixel (new_pixel), .iterated_all (iterated_all), .done (done), .pre_center_addr (pre_center_addr), .upleft_addr (upleft_addr), .up_addr (up_addr), .upright_addr (upright_addr), .left_addr (left_addr), .right_addr (right_addr), .downleft_addr (downleft_addr), .down_addr (down_addr), .downright_addr (downright_addr), .sel_row (sel_row), .sel_col (sel_col), .strb_value (strb_value) ); endmodule // eda_regional_max

## eda_controller

Table 4.2 eda_controller code

`include "eda_global_define.svh" module eda_controller #( // synopsys template parameter PIXEL_WIDTH = `CFG_PIXEL_WIDTH, parameter WINDOW_WIDTH = `CFG_WINDOW_WIDTH, parameter ADDR_WIDTH = `CFG_ADDR_WIDTH, parameter I_WIDTH = `CFG_I_WIDTH, parameter J_WIDTH = `CFG_J_WIDTH ) ( // Port Declarations input wire clk, // Clock signal input wire reset_n, // Asynchronous reset, active LOW input wire start, // Start input wire iterated_all, // Check if all pixels have been iterated input wire [WINDOW_WIDTH-2:0] fifo_empty, // FIFO empty input wire [WINDOW_WIDTH-2:0] push_positions, // Positions need to push to FIFO input wire [ADDR_WIDTH-1:0] data_out, // Data out from FIFO input wire [I_WIDTH - 1:0] next_row, // Next row input wire [J_WIDTH - 1:0] next_col, // Next column output reg new_pixel, // Iterate new pixel output wire [WINDOW_WIDTH-2:0] read_en, // Read FIOFO enable output reg clear, // Clear all RAMs output reg [ADDR_WIDTH-1:0] center_addr, // Center address output reg [ADDR_WIDTH-1:0] pre_center_addr, output reg update_strb, // Clear strobe output reg done // Done for an image ); // Internal Declarations // Declare any pre-registered internal signals reg new_pixel_cld; reg done_cld; // Module Declarations reg update_addr; // Update address reg extend_addr; wire check_next; // Check when can jump to {next_row, next_col} reg pre_new_pixel; // State encoding parameter ST_IDLE = 3'd0, ST_START = 3'd1, ST_NEXT = 3'd2, ST_DONE = 3'd3, ST_EXTEND = 3'd4; reg [2:0] current_state, next_state; //----------------------------------------------------------------- // Next State Block for machine csm //----------------------------------------------------------------- always @( check_next, current_state, iterated_all, start ) begin : next_state_block_proc case (current_state) ST_IDLE: begin if (start) next_state = ST_START; else next_state = ST_IDLE; end ST_START: begin if (iterated_all) next_state = ST_DONE; else if (check_next) next_state = ST_NEXT; else next_state = ST_EXTEND; end ST_NEXT: begin if (iterated_all) next_state = ST_DONE; else if (!check_next) next_state = ST_EXTEND; else next_state = ST_START; end ST_DONE: begin if (start) next_state = ST_START; else next_state = ST_DONE; end ST_EXTEND: begin if (iterated_all) next_state = ST_DONE; else if (check_next) next_state = ST_NEXT; else next_state = ST_START; end default: next_state = ST_IDLE; endcase end // Next State Block //----------------------------------------------------------------- // Output Block for machine csm //----------------------------------------------------------------- always @( check_next, current_state, iterated_all, start ) begin : output_block_proc // Default Assignment clear = 0; update_strb = 0; // Default Assignment To Internals update_addr = 0; extend_addr = 0; pre_new_pixel = 0; // Combined Actions case (current_state) ST_START: begin if (iterated_all) begin end else if (check_next) begin update_addr = 1; update_strb = 1; end else extend_addr = 1; end ST_NEXT: begin update_addr = 1; update_strb = 1; if (iterated_all) begin end else if (!check_next) begin extend_addr = 1; update_strb = 0; update_addr = 0; end end ST_DONE: begin if (start) clear = 1; end ST_EXTEND: begin extend_addr = 1; if (iterated_all) begin end else if (check_next) begin extend_addr = 0; update_strb = 1; update_addr = 1; end end endcase end // Output Block //----------------------------------------------------------------- // Clocked Block for machine csm //----------------------------------------------------------------- always @( posedge clk, negedge reset_n ) begin : clocked_block_proc if (!reset_n) begin current_state <= ST_IDLE; // Reset Values new_pixel_cld <= 0; done_cld <= 0; end else begin current_state <= next_state; // Combined Actions case (current_state) ST_IDLE: begin if (start) new_pixel_cld <= 1; end ST_START: begin if (iterated_all) begin end else if (check_next) new_pixel_cld <= (!done_cld); else new_pixel_cld <= (!done_cld); end ST_NEXT: begin if (iterated_all) begin end else if (!check_next) new_pixel_cld <= (!done_cld); end ST_DONE: begin done_cld <= 1; new_pixel_cld <= 0; if (start) done_cld <= 0; end ST_EXTEND: begin if (iterated_all) begin end else if (check_next) new_pixel_cld <= (!done_cld); end endcase end end // Clocked Block // Concurrent Statements // Clocked output assignments always @( new_pixel_cld, done_cld ) begin : clocked_output_proc new_pixel = new_pixel_cld; done = done_cld; end // pre_center_addr always @(*) begin : proc_pre_center_addr pre_center_addr = 0; if (extend_addr) begin pre_center_addr = data_out; end else if (update_addr) begin pre_center_addr = {next_row, next_col}; end end // center_addr always @(posedge clk or negedge reset_n) begin : proc_center_addr if(~reset_n) begin center_addr <= 0; end else begin center_addr <= pre_center_addr; end end // read_en assign read_en = ((fifo_empty + 1) & (~fifo_empty)); // check_next assign check_next = ((push_positions == 0) & (fifo_empty == {(WINDOW_WIDTH-1){1'b1}})); endmodule // eda_controller

## eda_compare

Table 4.3 eda_compare code

`include "eda_global_define.svh" module eda_compare #( parameter M = `CFG_M , parameter N = `CFG_N , parameter PIXEL_WIDTH = `CFG_PIXEL_WIDTH , parameter WINDOW_WIDTH = `CFG_WINDOW_WIDTH , parameter ADDR_WIDTH = `CFG_ADDR_WIDTH )( input clk , input reset_n , input new_pixel , input [PIXEL_WIDTH * WINDOW_WIDTH - 1:0] window_values , input [WINDOW_WIDTH - 2:0] neigh_addr_valid, input [WINDOW_WIDTH - 2:0] iterated_idx , output logic compare_out , output [WINDOW_WIDTH - 2:0] push_positions ); logic [WINDOW_WIDTH - 2:0] equal_positions; //Find max value in 9 pixels logic [PIXEL_WIDTH - 1:0] max_value ; logic [PIXEL_WIDTH - 1:0] value_l1[0:3]; generate for (genvar i = 0; i < 8; i = i + 2) begin eda_max cl1 ( .a (window_values[i * PIXEL_WIDTH + 7 -: PIXEL_WIDTH] ), .b (window_values[(i + 1) * PIXEL_WIDTH + 7 -: PIXEL_WIDTH]), .out (value_l1[i / 2] ) ); end endgenerate logic [PIXEL_WIDTH - 1:0] value_l2[0:1]; generate for (genvar i = 0; i < 4; i = i + 2) begin eda_max cl2 ( .a (value_l1[i] ), .b (value_l1[i + 1]), .out (value_l2[i / 2]) ); end endgenerate logic [PIXEL_WIDTH - 1:0] value_l3[0:0]; generate for (genvar i = 0; i < 2; i = i + 2) begin eda_max cl3 ( .a (value_l2[i] ), .b (value_l2[i + 1]), .out (value_l3[i / 2]) ); end endgenerate eda_max clfinal ( .a (value_l3[0] ), .b (window_values[PIXEL_WIDTH * WINDOW_WIDTH - 1 -: PIXEL_WIDTH]), .out (max_value ) ); always_comb begin if (max_value > window_values[4 * PIXEL_WIDTH + 7 -: PIXEL_WIDTH]) begin compare_out = 0; end else begin compare_out = 1; end end // Generate equal_positions generate for (genvar i = 0; i < WINDOW_WIDTH; i++) begin if(i < 4) begin always_comb begin if(window_values[i * PIXEL_WIDTH + 7 : i * PIXEL_WIDTH] == window_values[4 * PIXEL_WIDTH + 7 -: PIXEL_WIDTH]) begin equal_positions[i] = new_pixel; end else begin equal_positions[i] = 0; end end end else if(i > 4) begin always_comb begin if(window_values[i * PIXEL_WIDTH + 7 : i * PIXEL_WIDTH] == window_values[4 * PIXEL_WIDTH + 7 -: PIXEL_WIDTH]) begin equal_positions[i - 1] = new_pixel; end else begin equal_positions[i - 1] = 0; end end end end endgenerate assign push_positions = equal_positions & (~iterated_idx) & neigh_addr_valid; endmodule

## eda_fifos

Table 4.4 eda_fifos code

`include "eda_global_define.svh" module eda_fifos #( parameter M = `CFG_M , parameter N = `CFG_N , parameter PIXEL_WIDTH = `CFG_PIXEL_WIDTH , parameter WINDOW_WIDTH = `CFG_WINDOW_WIDTH, parameter ADDR_WIDTH = `CFG_ADDR_WIDTH , parameter I_WIDTH = `CFG_I_WIDTH , parameter J_WIDTH = `CFG_J_WIDTH , parameter FIFO_DEPTH = `CFG_FIFO_DEPTH , parameter DATA_WIDTH = `CFG_DATA_WIDTH )( input clk , input reset_n , input [WINDOW_WIDTH - 2:0] read_en , input [WINDOW_WIDTH - 2:0] push_positions , input [ADDR_WIDTH - 1:0] upleft_addr , input [ADDR_WIDTH - 1:0] up_addr , input [ADDR_WIDTH - 1:0] upright_addr , input [ADDR_WIDTH - 1:0] left_addr , input [ADDR_WIDTH - 1:0] right_addr , input [ADDR_WIDTH - 1:0] downleft_addr , input [ADDR_WIDTH - 1:0] down_addr , input [ADDR_WIDTH - 1:0] downright_addr , output [WINDOW_WIDTH - 2:0] fifo_empty , output [ADDR_WIDTH - 1:0] data_out ); //¦----------¦--------¦-----------¦ //¦ upleft/7 ¦ up/6 ¦ upright/5 ¦ //¦----------¦--------¦-----------¦ //¦ left/4 ¦ center ¦ right/3 ¦ //¦----------¦--------¦-----------¦ //¦downleft/2¦ down/1 ¦downright/0¦ //¦----------¦--------¦-----------¦ logic inv_clk; logic upleft_read_en; logic up_read_en; logic upright_read_en; logic left_read_en; logic right_read_en; logic downleft_read_en; logic down_read_en; logic downright_read_en; logic upleft_empty; logic up_empty; logic upright_empty; logic left_empty; logic right_empty; logic downleft_empty; logic down_empty; logic downright_empty; logic upleft_push_position; logic up_push_position; logic upright_push_position; logic left_push_position; logic right_push_position; logic downleft_push_position; logic down_push_position; logic downright_push_position; logic [ADDR_WIDTH - 1:0] upleft_data_out; logic [ADDR_WIDTH - 1:0] up_data_out; logic [ADDR_WIDTH - 1:0] upright_data_out; logic [ADDR_WIDTH - 1:0] left_data_out; logic [ADDR_WIDTH - 1:0] right_data_out; logic [ADDR_WIDTH - 1:0] downleft_data_out; logic [ADDR_WIDTH - 1:0] down_data_out; logic [ADDR_WIDTH - 1:0] downright_data_out; logic [ADDR_WIDTH - 1:0] pre_data_out; assign {upleft_read_en , up_read_en , upright_read_en , left_read_en , right_read_en , downleft_read_en, down_read_en, downright_read_en } = read_en; assign {upleft_push_position , up_push_position , upright_push_position , left_push_position , right_push_position , downleft_push_position, down_push_position, downright_push_position } = push_positions; assign fifo_empty = {upleft_empty , up_empty , upright_empty , left_empty , right_empty , downleft_empty, down_empty, downright_empty }; assign data_out = (!downright_empty) ? downright_data_out : (!down_empty ) ? down_data_out : (!downleft_empty ) ? downleft_data_out : (!right_empty ) ? right_data_out : (!left_empty ) ? left_data_out : (!upright_empty ) ? upright_data_out : (!up_empty ) ? up_data_out : (!upleft_empty ) ? upleft_data_out : pre_data_out ; always_comb begin : proc_pre_data_out pre_data_out = 0; if (downright_push_position) begin pre_data_out = downright_addr; end else if (down_push_position) begin pre_data_out = down_addr; end else if (downleft_push_position) begin pre_data_out = downleft_addr; end else if (right_push_position) begin pre_data_out = right_addr; end else if (left_push_position) begin pre_data_out = left_addr; end else if (upright_push_position) begin pre_data_out = upright_addr; end else if (up_push_position) begin pre_data_out = up_addr; end else if (upleft_push_position) begin pre_data_out = upleft_addr; end end assign inv_clk = (~clk); sync_fifo #( .FIFO_DEPTH(FIFO_DEPTH ) , // FIFO depth .DATA_WIDTH(DATA_WIDTH ) , // Data width .ADDR_WIDTH($clog2(FIFO_DEPTH) ) // Address width ) sync_fifo_upleft ( .i_clk (inv_clk ), // Clock signal .i_rst_n (reset_n ), // Source domain asynchronous reset (active low) .i_valid_s (upleft_push_position ), // Request write data into FIFO .i_almostfull_lvl ($clog2(FIFO_DEPTH)'(FIFO_DEPTH-2) ), // The number of empty memory locations in the FIFO at which the o_almostfull flag is active .i_datain (upleft_addr ), // Push data in FIFO .i_ready_m (upleft_read_en ), // Request read data from FIFO .i_almostempty_lvl ($clog2(FIFO_DEPTH)'('b10) ), // The number of empty memory locations in the FIFO at which the o_almostempty flag is active .o_ready_s ( ), // Status write data into FIFO (if FIFO not full then o_ready_s = 1) .o_almostfull ( ), // FIFO almostfull flag (determined by i_almostfull_lvl) .o_full ( ), // FIFO full flag .o_valid_m ( ), // Status read data from FIFO (if FIFO not empty then o_valid_m = 1) .o_almostempty ( ), // FIFO almostempty flag (determined by i_almostempty_lvl) .o_empty (upleft_empty ), // FIFO empty flag .o_dataout (upleft_data_out ) // Pop data from FIFO ); sync_fifo #( .FIFO_DEPTH(FIFO_DEPTH ) , // FIFO depth .DATA_WIDTH(DATA_WIDTH ) , // Data width .ADDR_WIDTH($clog2(FIFO_DEPTH) ) // Address width ) sync_fifo_up ( .i_clk (inv_clk ), // Clock signal .i_rst_n (reset_n ), // Source domain asynchronous reset (active low) .i_valid_s (up_push_position ), // Request write data into FIFO .i_almostfull_lvl ($clog2(FIFO_DEPTH)'(FIFO_DEPTH-2) ), // The number of empty memory locations in the FIFO at which the o_almostfull flag is active .i_datain (up_addr ), // Push data in FIFO .i_ready_m (up_read_en ), // Request read data from FIFO .i_almostempty_lvl ($clog2(FIFO_DEPTH)'('b10) ), // The number of empty memory locations in the FIFO at which the o_almostempty flag is active .o_ready_s ( ), // Status write data into FIFO (if FIFO not full then o_ready_s = 1) .o_almostfull ( ), // FIFO almostfull flag (determined by i_almostfull_lvl) .o_full ( ), // FIFO full flag .o_valid_m ( ), // Status read data from FIFO (if FIFO not empty then o_valid_m = 1) .o_almostempty ( ), // FIFO almostempty flag (determined by i_almostempty_lvl) .o_empty (up_empty ), // FIFO empty flag .o_dataout (up_data_out ) // Pop data from FIFO ); sync_fifo #( .FIFO_DEPTH(FIFO_DEPTH ) , // FIFO depth .DATA_WIDTH(DATA_WIDTH ) , // Data width .ADDR_WIDTH($clog2(FIFO_DEPTH) ) // Address width ) sync_fifo_upright ( .i_clk (inv_clk ), // Clock signal .i_rst_n (reset_n ), // Source domain asynchronous reset (active low) .i_valid_s (upright_push_position ), // Request write data into FIFO .i_almostfull_lvl ($clog2(FIFO_DEPTH)'(FIFO_DEPTH-2) ), // The number of empty memory locations in the FIFO at which the o_almostfull flag is active .i_datain (upright_addr ), // Push data in FIFO .i_ready_m (upright_read_en ), // Request read data from FIFO .i_almostempty_lvl ($clog2(FIFO_DEPTH)'('b10) ), // The number of empty memory locations in the FIFO at which the o_almostempty flag is active .o_ready_s ( ), // Status write data into FIFO (if FIFO not full then o_ready_s = 1) .o_almostfull ( ), // FIFO almostfull flag (determined by i_almostfull_lvl) .o_full ( ), // FIFO full flag .o_valid_m ( ), // Status read data from FIFO (if FIFO not empty then o_valid_m = 1) .o_almostempty ( ), // FIFO almostempty flag (determined by i_almostempty_lvl) .o_empty (upright_empty ), // FIFO empty flag .o_dataout (upright_data_out ) // Pop data from FIFO ); sync_fifo #( .FIFO_DEPTH(FIFO_DEPTH ) , // FIFO depth .DATA_WIDTH(DATA_WIDTH ) , // Data width .ADDR_WIDTH($clog2(FIFO_DEPTH) ) // Address width ) sync_fifo_left ( .i_clk (inv_clk ), // Clock signal .i_rst_n (reset_n ), // Source domain asynchronous reset (active low) .i_valid_s (left_push_position ), // Request write data into FIFO .i_almostfull_lvl ($clog2(FIFO_DEPTH)'(FIFO_DEPTH-2) ), // The number of empty memory locations in the FIFO at which the o_almostfull flag is active .i_datain (left_addr ), // Push data in FIFO .i_ready_m (left_read_en ), // Request read data from FIFO .i_almostempty_lvl ($clog2(FIFO_DEPTH)'('b10) ), // The number of empty memory locations in the FIFO at which the o_almostempty flag is active .o_ready_s ( ), // Status write data into FIFO (if FIFO not full then o_ready_s = 1) .o_almostfull ( ), // FIFO almostfull flag (determined by i_almostfull_lvl) .o_full ( ), // FIFO full flag .o_valid_m ( ), // Status read data from FIFO (if FIFO not empty then o_valid_m = 1) .o_almostempty ( ), // FIFO almostempty flag (determined by i_almostempty_lvl) .o_empty (left_empty ), // FIFO empty flag .o_dataout (left_data_out ) // Pop data from FIFO ); sync_fifo #( .FIFO_DEPTH(FIFO_DEPTH ) , // FIFO depth .DATA_WIDTH(DATA_WIDTH ) , // Data width .ADDR_WIDTH($clog2(FIFO_DEPTH) ) // Address width ) sync_fifo_right ( .i_clk (inv_clk ), // Clock signal .i_rst_n (reset_n ), // Source domain asynchronous reset (active low) .i_valid_s (right_push_position ), // Request write data into FIFO .i_almostfull_lvl ($clog2(FIFO_DEPTH)'(FIFO_DEPTH-2) ), // The number of empty memory locations in the FIFO at which the o_almostfull flag is active .i_datain (right_addr ), // Push data in FIFO .i_ready_m (right_read_en ), // Request read data from FIFO .i_almostempty_lvl ($clog2(FIFO_DEPTH)'('b10) ), // The number of empty memory locations in the FIFO at which the o_almostempty flag is active .o_ready_s ( ), // Status write data into FIFO (if FIFO not full then o_ready_s = 1) .o_almostfull ( ), // FIFO almostfull flag (determined by i_almostfull_lvl) .o_full ( ), // FIFO full flag .o_valid_m ( ), // Status read data from FIFO (if FIFO not empty then o_valid_m = 1) .o_almostempty ( ), // FIFO almostempty flag (determined by i_almostempty_lvl) .o_empty (right_empty ), // FIFO empty flag .o_dataout (right_data_out ) // Pop data from FIFO ); sync_fifo #( .FIFO_DEPTH(FIFO_DEPTH ) , // FIFO depth .DATA_WIDTH(DATA_WIDTH ) , // Data width .ADDR_WIDTH($clog2(FIFO_DEPTH) ) // Address width ) sync_fifo_downleft ( .i_clk (inv_clk ), // Clock signal .i_rst_n (reset_n ), // Source domain asynchronous reset (active low) .i_valid_s (downleft_push_position ), // Request write data into FIFO .i_almostfull_lvl ($clog2(FIFO_DEPTH)'(FIFO_DEPTH-2) ), // The number of empty memory locations in the FIFO at which the o_almostfull flag is active .i_datain (downleft_addr ), // Push data in FIFO .i_ready_m (downleft_read_en ), // Request read data from FIFO .i_almostempty_lvl ($clog2(FIFO_DEPTH)'('b10) ), // The number of empty memory locations in the FIFO at which the o_almostempty flag is active .o_ready_s ( ), // Status write data into FIFO (if FIFO not full then o_ready_s = 1) .o_almostfull ( ), // FIFO almostfull flag (determined by i_almostfull_lvl) .o_full ( ), // FIFO full flag .o_valid_m ( ), // Status read data from FIFO (if FIFO not empty then o_valid_m = 1) .o_almostempty ( ), // FIFO almostempty flag (determined by i_almostempty_lvl) .o_empty (downleft_empty ), // FIFO empty flag .o_dataout (downleft_data_out ) // Pop data from FIFO ); sync_fifo #( .FIFO_DEPTH(FIFO_DEPTH ) , // FIFO depth .DATA_WIDTH(DATA_WIDTH ) , // Data width .ADDR_WIDTH($clog2(FIFO_DEPTH) ) // Address width ) sync_fifo_down ( .i_clk (inv_clk ), // Clock signal .i_rst_n (reset_n ), // Source domain asynchronous reset (active low) .i_valid_s (down_push_position ), // Request write data into FIFO .i_almostfull_lvl ($clog2(FIFO_DEPTH)'(FIFO_DEPTH-2) ), // The number of empty memory locations in the FIFO at which the o_almostfull flag is active .i_datain (down_addr ), // Push data in FIFO .i_ready_m (down_read_en ), // Request read data from FIFO .i_almostempty_lvl ($clog2(FIFO_DEPTH)'('b10) ), // The number of empty memory locations in the FIFO at which the o_almostempty flag is active .o_ready_s ( ), // Status write data into FIFO (if FIFO not full then o_ready_s = 1) .o_almostfull ( ), // FIFO almostfull flag (determined by i_almostfull_lvl) .o_full ( ), // FIFO full flag .o_valid_m ( ), // Status read data from FIFO (if FIFO not empty then o_valid_m = 1) .o_almostempty ( ), // FIFO almostempty flag (determined by i_almostempty_lvl) .o_empty (down_empty ), // FIFO empty flag .o_dataout (down_data_out ) // Pop data from FIFO ); sync_fifo #( .FIFO_DEPTH(FIFO_DEPTH ) , // FIFO depth .DATA_WIDTH(DATA_WIDTH ) , // Data width .ADDR_WIDTH($clog2(FIFO_DEPTH) ) // Address width ) sync_fifo_downright ( .i_clk (inv_clk ), // Clock signal .i_rst_n (reset_n ), // Source domain asynchronous reset (active low) .i_valid_s (downright_push_position ), // Request write data into FIFO .i_almostfull_lvl ($clog2(FIFO_DEPTH)'(FIFO_DEPTH-2) ), // The number of empty memory locations in the FIFO at which the o_almostfull flag is active .i_datain (downright_addr ), // Push data in FIFO .i_ready_m (downright_read_en ), // Request read data from FIFO .i_almostempty_lvl ($clog2(FIFO_DEPTH)'('b10) ), // The number of empty memory locations in the FIFO at which the o_almostempty flag is active .o_ready_s ( ), // Status write data into FIFO (if FIFO not full then o_ready_s = 1) .o_almostfull ( ), // FIFO almostfull flag (determined by i_almostfull_lvl) .o_full ( ), // FIFO full flag .o_valid_m ( ), // Status read data from FIFO (if FIFO not empty then o_valid_m = 1) .o_almostempty ( ), // FIFO almostempty flag (determined by i_almostempty_lvl) .o_empty (downright_empty ), // FIFO empty flag .o_dataout (downright_data_out ) // Pop data from FIFO ); endmodule : eda_fifos

## eda_img_ram

Table 4.5 eda_img_ram code

`include "eda_global_define.svh" module eda_img_ram #( parameter M = `CFG_M , parameter N = `CFG_N , parameter PIXEL_WIDTH = `CFG_PIXEL_WIDTH , parameter WINDOW_WIDTH = `CFG_WINDOW_WIDTH , parameter ADDR_WIDTH = `CFG_ADDR_WIDTH , parameter I_WIDTH = `CFG_I_WIDTH , parameter J_WIDTH = `CFG_J_WIDTH )( input clk , input reset_n , input write_en , input [ADDR_WIDTH - 1:0] wr_addr , input [PIXEL_WIDTH- 1:0] pixel_in , input [ADDR_WIDTH - 1:0] center_addr , //{i, j} output logic [PIXEL_WIDTH * WINDOW_WIDTH - 1:0] window_values , output logic [WINDOW_WIDTH - 2:0] neigh_addr_valid, output [ADDR_WIDTH - 1:0] upleft_addr , output [ADDR_WIDTH - 1:0] up_addr , output [ADDR_WIDTH - 1:0] upright_addr , output [ADDR_WIDTH - 1:0] left_addr , output [ADDR_WIDTH - 1:0] right_addr , output [ADDR_WIDTH - 1:0] downleft_addr , output [ADDR_WIDTH - 1:0] down_addr , output [ADDR_WIDTH - 1:0] downright_addr ); logic [M - 1:0][N - 1:0][PIXEL_WIDTH - 1:0] img_memory; //¦----------¦--------¦-----------¦ //¦ upleft ¦ up ¦ upright ¦ //¦----------¦--------¦-----------¦ //¦ left ¦ center ¦ right ¦ //¦----------¦--------¦-----------¦ //¦ downleft ¦ down ¦ downright ¦ //¦----------¦--------¦-----------¦ // Write pixel into memory logic [I_WIDTH - 1:0] i_pixel; logic [J_WIDTH - 1:0] j_pixel; assign i_pixel = wr_addr[ADDR_WIDTH - 1:J_WIDTH]; assign j_pixel = wr_addr[J_WIDTH - 1:0] ; always_ff @(posedge clk or negedge reset_n) begin if(~reset_n) begin img_memory <= 0; end else if(write_en) begin img_memory[i_pixel][j_pixel] <= pixel_in; end end // Generate neighborhood address of center address logic [I_WIDTH - 1:0] i_center; logic [J_WIDTH - 1:0] j_center; logic [I_WIDTH - 1:0] i_center_minus; logic [I_WIDTH - 1:0] i_center_plus ; logic [J_WIDTH - 1:0] j_center_plus ; logic [J_WIDTH - 1:0] j_center_minus; assign i_center = center_addr[ADDR_WIDTH - 1:J_WIDTH]; assign j_center = center_addr[J_WIDTH - 1:0] ; assign i_center_minus = i_center - 1; assign i_center_plus = i_center + 1; assign j_center_plus = j_center + 1; assign j_center_minus = j_center - 1; assign upleft_addr = {i_center_minus, j_center_minus}; assign up_addr = {i_center_minus, j_center }; assign upright_addr = {i_center_minus, j_center_plus }; assign left_addr = {i_center , j_center_minus}; assign right_addr = {i_center , j_center_plus }; assign downleft_addr = {i_center_plus , j_center_minus}; assign down_addr = {i_center_plus , j_center }; assign downright_addr = {i_center_plus , j_center_plus }; // assign neigh_addr = {upleft_addr, up_addr, upright_addr, left_addr, right_addr, downleft_addr, down_addr, downright_addr}; // Generate neighborhood address valid always_comb begin if(i_center == 0) begin if(j_center == 0) begin neigh_addr_valid = 8'b00001011; end else if(j_center == N - 1) begin neigh_addr_valid = 8'b00010110; end else begin neigh_addr_valid = 8'b00011111; end end else if(i_center == M - 1) begin if(j_center == 0) begin neigh_addr_valid = 8'b01101000; end else if(j_center == N - 1) begin neigh_addr_valid = 8'b11010000; end else begin neigh_addr_valid = 8'b11111000; end end else if(j_center == 0) begin neigh_addr_valid = 8'b01101011; end else if(j_center == N - 1) begin neigh_addr_valid = 8'b11010110; end else begin neigh_addr_valid = 8'b11111111; end end // Assign window values logic [PIXEL_WIDTH * WINDOW_WIDTH - 1:0] window_values_real; logic [WINDOW_WIDTH :0] addr_valid ; assign addr_valid = {neigh_addr_valid[7:4], 1'b1, neigh_addr_valid[3:0]}; assign window_values_real = {img_memory[i_center_minus][j_center_minus], img_memory[i_center_minus][j_center ], img_memory[i_center_minus][j_center_plus], img_memory[i_center ][j_center_minus], img_memory[i_center ][j_center ], img_memory[i_center ][j_center_plus], img_memory[i_center_plus ][j_center_minus], img_memory[i_center_plus ][j_center ], img_memory[i_center_plus ][j_center_plus]}; generate for (genvar i = 0; i < WINDOW_WIDTH; i++) begin always_comb begin if(addr_valid[i]) begin window_values[i * PIXEL_WIDTH + 7 -: 8] = window_values_real[i * PIXEL_WIDTH + 7 -: 8]; end else begin window_values[i * PIXEL_WIDTH + 7 -: 8] = 0; end end end endgenerate endmodule

## eda_iterated_ram

Table 4.6 eda_iterated_ram code

`include "eda_global_define.svh" module eda_iterated_ram #( parameter M = `CFG_M , parameter N = `CFG_N , parameter WINDOW_WIDTH = `CFG_WINDOW_WIDTH , parameter ADDR_WIDTH = `CFG_ADDR_WIDTH , parameter I_WIDTH = `CFG_I_WIDTH , parameter J_WIDTH = `CFG_J_WIDTH )( input clk , input reset_n , input clear , input new_pixel , input done , input [WINDOW_WIDTH - 2:0] fifo_empty , input [ADDR_WIDTH - 1:0] center_addr , //{i, j} input [ADDR_WIDTH - 1:0] upleft_addr , input [ADDR_WIDTH - 1:0] up_addr , input [ADDR_WIDTH - 1:0] upright_addr , input [ADDR_WIDTH - 1:0] left_addr , input [ADDR_WIDTH - 1:0] right_addr , input [ADDR_WIDTH - 1:0] downleft_addr , input [ADDR_WIDTH - 1:0] down_addr , input [ADDR_WIDTH - 1:0] downright_addr , input [WINDOW_WIDTH - 2:0] neigh_addr_valid, input [WINDOW_WIDTH - 2:0] push_positions , output logic [WINDOW_WIDTH - 2:0] iterated_idx , output logic [I_WIDTH - 1:0] next_row , output logic [J_WIDTH - 1:0] next_col , output logic [M - 1:0] sel_row , // Find next row (Ex: 00010..00) output logic [M - 1:0][N - 1:0] sel_col , // Find next column (Ex: 00010..00) for all rows output logic iterated_all ); logic inv_clk ; logic have_done ; logic [M - 1:0][N - 1:0] iterated_memory; logic [WINDOW_WIDTH - 2:0][ADDR_WIDTH - 1:0] addr_arr ; logic [M - 1:0][N - 1:0] current_row ; // Get data from all rows of iterated RAM logic [M - 1:0] combine_row ; // Check if each row has pixel that has not iterated yet logic [N - 1:0] final_sel_col ; // Find next column (Ex: 00010..00) assign inv_clk = (~clk); // Next row eda_one_hot_to_bin #( .ONE_HOT_WIDTH (M ), .BIN_WIDTH (I_WIDTH) ) eda_one_hot_to_bin_next_row ( .one_hot_code (sel_row ), .bin_code (next_row) ); // Next column eda_one_hot_to_bin #( .ONE_HOT_WIDTH (N ), .BIN_WIDTH (J_WIDTH) ) eda_one_hot_to_bin_next_col ( .one_hot_code (final_sel_col ), .bin_code (next_col ) ); // Get data from all rows of iterated RAM generate for (genvar i = 0; i < M; i++) begin for (genvar j = 0; j < N; j++) begin assign current_row[i][j] = iterated_memory[i][j]; end end endgenerate // Find next column (Ex: 00010..00) generate for (genvar i = 0; i < M; i++) begin assign sel_col[i] = ((current_row[i] + 1) & (~current_row[i])); end endgenerate // Check if each row has pixel that has not iterated yet generate for (genvar i = 0; i < M; i++) begin assign combine_row[i] = ¦sel_col[i]; end endgenerate // Select row assign sel_row = (((~combine_row) + 1) & combine_row); // Select column always_comb begin final_sel_col = 0; for (int i = 0; i < M; i++) begin if (sel_row[i] == 1) begin final_sel_col = sel_col[i]; end end end // Check if all pixels have been iterated always_ff @(posedge clk or negedge reset_n) begin : proc_iterated_all if(~reset_n) begin iterated_all <= 0; end else begin iterated_all <= ((final_sel_col == 0) & (sel_row == 0)) & (fifo_empty == {(WINDOW_WIDTH-1){1'b1}}) & new_pixel; end end assign addr_arr = {upleft_addr, up_addr, upright_addr, left_addr, right_addr, downleft_addr, down_addr, downright_addr}; always_ff @(posedge clk or negedge reset_n) begin : proc_have_done if(~reset_n) begin have_done <= 0; end else begin if (!have_done) begin have_done <= done; end else begin have_done <= 0; end end end always_ff @(posedge inv_clk or negedge reset_n) begin if(~reset_n) begin iterated_memory <= 0; end else if(have_done) begin iterated_memory <= 0; end else begin for (int i = 0; i < WINDOW_WIDTH - 1; i++) begin if(push_positions[i]) begin iterated_memory[addr_arr[i][ADDR_WIDTH - 1:J_WIDTH]][addr_arr[i][J_WIDTH - 1:0]] <= 1; end end if (new_pixel) begin iterated_memory[center_addr[ADDR_WIDTH - 1:J_WIDTH]][center_addr[J_WIDTH - 1:0]] <= 1; end end end // genvarerate ouput generate for (genvar i = 0; i < WINDOW_WIDTH - 1; i++) begin always_comb begin if (neigh_addr_valid[i]) begin iterated_idx[i] = iterated_memory[addr_arr[i][ADDR_WIDTH - 1:J_WIDTH]][addr_arr[i][J_WIDTH - 1:0]]; end else begin iterated_idx[i] = 0; end end end endgenerate endmodule

## eda_strobe_ram

Table 4.7 eda_strobe_ram code

`include "eda_global_define.svh" module eda_strobe_ram #( parameter M = `CFG_M , parameter N = `CFG_N , parameter WINDOW_WIDTH = `CFG_WINDOW_WIDTH , parameter ADDR_WIDTH = `CFG_ADDR_WIDTH , parameter I_WIDTH = `CFG_I_WIDTH , parameter J_WIDTH = `CFG_J_WIDTH ) ( input clk , input reset_n , input update_strb , input clear , input new_pixel , input iterated_all , input done , input [ADDR_WIDTH - 1:0] pre_center_addr , input [ADDR_WIDTH - 1:0] upleft_addr , input [ADDR_WIDTH - 1:0] up_addr , input [ADDR_WIDTH - 1:0] upright_addr , input [ADDR_WIDTH - 1:0] left_addr , input [ADDR_WIDTH - 1:0] right_addr , input [ADDR_WIDTH - 1:0] downleft_addr , input [ADDR_WIDTH - 1:0] down_addr , input [ADDR_WIDTH - 1:0] downright_addr , input [M-1:0] sel_row , // Find next row (Ex: 00010..00) input [M-1:0][N-1:0] sel_col , // Find next column (Ex: 00010..00) for all rows output [M-1:0][N-1:0] strb_value ); logic [M - 1:0][N - 1:0] strb_memory; logic [WINDOW_WIDTH - 2:0][ADDR_WIDTH - 1:0] addr_arr ; //¦----------¦--------¦-----------¦ //¦ upleft ¦ up ¦ upright ¦ //¦----------¦--------¦-----------¦ //¦ left ¦ center ¦ right ¦ //¦----------¦--------¦-----------¦ //¦ downleft ¦ down ¦ downright ¦ //¦----------¦--------¦-----------¦ assign addr_arr = {upleft_addr, up_addr, upright_addr, left_addr, right_addr, downleft_addr, down_addr, downright_addr}; generate for (genvar i = 0; i < M; i = i + 1) begin for (genvar j = 0; j < N; j = j + 1) begin assign strb_value[i][j] = strb_memory[i][j]; end end endgenerate always_ff @(posedge clk or negedge reset_n) begin if(~reset_n) begin for (int i = 0; i < M; i++) begin for (int j = 0; j < N; j++) begin if ((i == 0) & (j == 0)) begin strb_memory[i][j] <= 1; end else begin strb_memory[i][j] <= 0; end end end end else if (clear) begin for (int i = 0; i < M; i++) begin for (int j = 0; j < N; j++) begin if ((i == 0) & (j == 0)) begin strb_memory[i][j] <= 1; end else begin strb_memory[i][j] <= 0; end end end end else if (update_strb & new_pixel & (!iterated_all)) begin for (int i = 0; i < M; i++) begin for (int j = 0; j < N; j++) begin if ((i == 0) & (j == 0)) begin strb_memory[i][j] <= 0; end else begin if (sel_row[i] & sel_col[i][j]) begin strb_memory[i][j] <= 1; end else begin strb_memory[i][j] <= 0; end end end end end else if ((!update_strb) & new_pixel & (!iterated_all)) begin strb_memory[pre_center_addr[ADDR_WIDTH-1:J_WIDTH]][pre_center_addr[J_WIDTH-1:0]] <= 1; end end endmodule

## eda_output_ram

Table 4.8 eda_output_ram code

`include "eda_global_define.svh" module eda_output_ram #( parameter M = `CFG_M , parameter N = `CFG_N , parameter PIXEL_WIDTH = `CFG_PIXEL_WIDTH , parameter WINDOW_WIDTH = `CFG_WINDOW_WIDTH , parameter ADDR_WIDTH = `CFG_ADDR_WIDTH , parameter I_WIDTH = `CFG_I_WIDTH , parameter J_WIDTH = `CFG_J_WIDTH )( input clk , // Clock input reset_n , // Asynchronous reset active low input clear , // Synchronous reset active low, use when load new image input new_pixel , input update_strb , input compare_out , input [M-1:0][N-1:0] strb_value , output logic [M-1:0][N-1:0] matrix_output ); logic compare_out_tmp; // logic iterated_all_reg; always_ff @(posedge clk or negedge reset_n) begin : proc_compare_out_tmp if(~reset_n) begin compare_out_tmp <= 1; end else begin if (clear ¦ update_strb) begin compare_out_tmp <= 1; end else if ((!compare_out) & new_pixel) begin compare_out_tmp <= 0; end end end genvar i, j; generate for (i = 0; i < M; i = i+1) begin for (j = 0; j < N; j = j+1) begin always_ff @(posedge clk or negedge reset_n) begin if(~reset_n) begin matrix_output[i][j] <= 1'b1; end else begin // clear matrix, using when load new image if (clear) begin matrix_output[i][j] <= 1'b1; end // Update pixels which has strb else if (update_strb & new_pixel) begin if (strb_value[i][j]) begin matrix_output[i][j] <= (compare_out & compare_out_tmp); end end else if (new_pixel) begin if (strb_value[i][j]) begin matrix_output[i][j] <= compare_out_tmp; end end end end end end endgenerate endmodule

## eda_max

Table 4.9 eda_max code

`include "eda_global_define.svh" module eda_max #( parameter PIXEL_WIDTH = `CFG_PIXEL_WIDTH )( input [PIXEL_WIDTH - 1:0] a , input [PIXEL_WIDTH - 1:0] b , output logic [PIXEL_WIDTH - 1:0] out ); always_comb begin if (a > b) out = a; else out = b; end endmodule

## eda_one_hot_to_bin

Table 4.10 eda_one_hot_to_bin code

`include "eda_global_define.svh" module eda_one_hot_to_bin #( parameter ONE_HOT_WIDTH = `CFG_N , parameter BIN_WIDTH = `CFG_J_WIDTH ) ( input [ONE_HOT_WIDTH - 1:0] one_hot_code, // One-hot code output logic [BIN_WIDTH - 1:0] bin_code // Binary code ); always_comb begin bin_code = 0; for (int i = ONE_HOT_WIDTH - 1; i >= 0; i--) begin if (one_hot_code[i]) begin bin_code = i; end end end endmodule

## sync_fifo

Table 4.11 sync_fifo code

`include "eda_global_define.svh" module sync_fifo #( parameter FIFO_DEPTH = `CFG_FIFO_DEPTH , // FIFO depth parameter DATA_WIDTH = `CFG_DATA_WIDTH , // Data width parameter ADDR_WIDTH = $clog2(FIFO_DEPTH) // Address width ) ( input i_clk , // Clock signal input i_rst_n , // Source domain asynchronous reset (active low) input i_valid_s , // Request write data into FIFO input [ADDR_WIDTH-1:0] i_almostfull_lvl , // The number of empty memory locations in the FIFO at which the o_almostfull flag is active input [DATA_WIDTH-1:0] i_datain , // Push data in FIFO input i_ready_m , // Request read data from FIFO input [ADDR_WIDTH-1:0] i_almostempty_lvl, // The number of empty memory locations in the FIFO at which the o_almostempty flag is active output o_ready_s , // Status write data into FIFO (if FIFO not full then o_ready_s = 1) output o_almostfull , // FIFO almostfull flag (determined by i_almostfull_lvl) output o_full , // FIFO full flag output o_valid_m , // Status read data from FIFO (if FIFO not empty then o_valid_m = 1) output o_almostempty , // FIFO almostempty flag (determined by i_almostempty_lvl) output o_empty , // FIFO empty flag output [DATA_WIDTH-1:0] o_dataout // Pop data from FIFO ); //============================================ // Internal signals and variables //============================================ wire [ADDR_WIDTH:0] wr_addr; // Write address (Write pointer) wire [ADDR_WIDTH:0] rd_addr; // Read address (Read pointer) wire wr_en ; // Write enable //============================================ // Synchronous FIFO memory //============================================ sync_fifo_mem sync_fifo_mem_inst ( .clk (i_clk ), // .reset_n(i_rst_n ), .wr_data(i_datain ), .wr_addr(wr_addr[ADDR_WIDTH-1:0]), .wr_en (wr_en ), .rd_addr(rd_addr[ADDR_WIDTH-1:0]), .rd_data(o_dataout ) ); //============================================ // Write control //============================================ write_control write_control_inst ( .clk (i_clk ), .reset_n (i_rst_n ), .wr_valid(i_valid_s), .wr_full (o_full ), .wr_en (wr_en ), .wr_addr (wr_addr ) ); //============================================ // Read control //============================================ read_control read_control_inst ( .clk (i_clk ), .reset_n (i_rst_n ), .rd_ready(i_ready_m), .rd_empty(o_empty ), .rd_addr (rd_addr ) ); //============================================ // Comparator //============================================ comparator comparator_inst ( .clk (i_clk ), .reset_n (i_rst_n ), .i_valid_s (i_valid_s ), .i_ready_m (i_ready_m ), .wr_addr (wr_addr ), .rd_addr (rd_addr ), .i_almostfull_lvl (i_almostfull_lvl ), .i_almostempty_lvl(i_almostempty_lvl), .o_ready_s (o_ready_s ), .o_almostfull (o_almostfull ), .o_full (o_full ), .o_valid_m (o_valid_m ), .o_almostempty (o_almostempty ), .o_empty (o_empty ) ); endmodule `include "eda_global_define.svh" module comparator #( parameter FIFO_DEPTH = `CFG_FIFO_DEPTH , // FIFO depth parameter ADDR_WIDTH = $clog2(`CFG_FIFO_DEPTH) // Address width ) ( input clk , // Clock signal input reset_n , // Source domain asynchronous reset (active low) input i_valid_s , // Request write data into FIFO input i_ready_m , // Request read data from FIFO input [ADDR_WIDTH:0] wr_addr , // Write address input [ADDR_WIDTH:0] rd_addr , // Read address input [ADDR_WIDTH-1:0] i_almostfull_lvl , // The number of empty memory locations in the FIFO at which the o_almostfull flag is active input [ADDR_WIDTH-1:0] i_almostempty_lvl, // The number of empty memory locations in the FIFO at which the o_almostempty flag is active output reg o_ready_s , // Status write data into FIFO (if FIFO not full then o_ready_s = 1) output reg o_almostfull , // FIFO almostfull flag (determined by i_almostfull_lvl) output reg o_full , // FIFO full flag output reg o_valid_m , // Status read data from FIFO (if FIFO not empty then o_valid_m = 1) output reg o_almostempty , // FIFO almostempty flag (determined by i_almostempty_lvl) output o_empty // FIFO empty flag ); //============================================ // Internal signals and variables //============================================ wire [ADDR_WIDTH:0] num_elements; // Number of elements //============================================ // Number of elements //============================================ assign num_elements = wr_addr + ((~rd_addr) + 1); // Number of elements = write address - read address //============================================ // Flags //============================================ // Flag FIFO almost full always @(posedge clk or negedge reset_n) begin : proc_o_almostfull if(~reset_n) begin o_almostfull <= 0; end else if ((num_elements == i_almostfull_lvl - 1) & i_valid_s & (!i_ready_m)) begin o_almostfull <= 1; end else if ((num_elements == i_almostfull_lvl) & i_ready_m & (!i_valid_s)) begin o_almostfull <= 0; end end // Flag FIFO full and flag valid for reading data always @(posedge clk or negedge reset_n) begin : proc_o_full if(~reset_n) begin o_full <= 0; o_ready_s <= 1; end else if ((((num_elements == (FIFO_DEPTH-1)) & i_valid_s) ¦ (num_elements == FIFO_DEPTH)) & (!i_ready_m)) begin o_full <= 1; o_ready_s <= 0; end else begin o_full <= 0; o_ready_s <= 1; end end // Flag FIFO almost empty always @(posedge clk or negedge reset_n) begin : proc_o_almostempty if(~reset_n) begin o_almostempty <= 1; end else if (((num_elements == i_almostempty_lvl + 1) & i_ready_m & (!i_valid_s)) ¦ o_empty) begin o_almostempty <= 1; end else if ((num_elements == i_almostempty_lvl) & i_valid_s & (!i_ready_m)) begin o_almostempty <= 0; end end // Flag FIFO empty always @(posedge clk or negedge reset_n) begin : proc_o_empty if(~reset_n) begin // o_empty <= 1; o_valid_m <= 0; end else if ((((num_elements == 1) & i_ready_m) ¦ (num_elements == 0)) & (!i_valid_s)) begin // o_empty <= 1; o_valid_m <= 0; end else begin // o_empty <= 0; o_valid_m <= 1; end end assign o_empty = (num_elements == 0); endmodule `include "eda_global_define.svh" module read_control #( parameter MEM_DEPTH = `CFG_FIFO_DEPTH , // Memory depth parameter DATA_WIDTH = `CFG_DATA_WIDTH , // Data width parameter ADDR_WIDTH = $clog2(MEM_DEPTH) // Address width ) ( input clk , // Clock signal input reset_n , // Source domain asynchronous reset (active low) input rd_ready, // Request read data from FIFO input rd_empty, // FIFO empty flag output reg [ADDR_WIDTH:0] rd_addr // Read address ); //============================================ // Internal signals and variables //============================================ wire rd_en; // Read enable //============================================ // Read address //============================================ always @(posedge clk or negedge reset_n) begin : proc_rd_addr if(~reset_n) begin rd_addr <= 0; end else begin if (rd_en & (rd_addr == ADDR_WIDTH'(MEM_DEPTH-1))) begin rd_addr <= 0; end else if (rd_en) begin rd_addr <= rd_addr + 1; end end end //============================================ // Read enable //============================================ assign rd_en = rd_ready & (!rd_empty); endmodule `include "eda_global_define.svh" module sync_fifo_mem #( parameter MEM_DEPTH = `CFG_FIFO_DEPTH , // Memory depth parameter DATA_WIDTH = `CFG_DATA_WIDTH , // Data width parameter ADDR_WIDTH = $clog2(MEM_DEPTH) // Address width ) ( input clk , // Clock signal // input reset_n, // Synchonous reset input [DATA_WIDTH-1:0] wr_data, // Write data input [ADDR_WIDTH-1:0] wr_addr, // Write address input wr_en , // Write enable input [ADDR_WIDTH-1:0] rd_addr, // Read address output [DATA_WIDTH-1:0] rd_data // Read data ); //============================================ // Internal signals and variables //============================================ reg [0:MEM_DEPTH-1][DATA_WIDTH-1:0] fifo_mem; // FIFO memory // Number of elements: MEM_DEPTH // Data width of each element: DATA_WIDTH //============================================ // Read data //============================================ assign rd_data = fifo_mem[rd_addr]; //============================================ // Write data to memory //============================================ always @(posedge clk) begin : proc_wr_data if (wr_en) begin fifo_mem[wr_addr] <= wr_data; end end endmodule `include "eda_global_define.svh" module write_control #( parameter MEM_DEPTH = `CFG_FIFO_DEPTH , // Memory depth parameter DATA_WIDTH = `CFG_DATA_WIDTH , // Data width parameter ADDR_WIDTH = $clog2(MEM_DEPTH) // Address width ) ( input clk , // Clock signal input reset_n , // Source domain asynchronous reset (active low) input wr_valid, // Request write data into FIFO input wr_full , // FIFO full flag output wr_en , // Write data output reg [ADDR_WIDTH:0] wr_addr // Write address ); //============================================ // Write address //============================================ always @(posedge clk or negedge reset_n) begin : proc_wr_addr if(~reset_n) begin wr_addr <= 0; end else begin if (wr_en & (wr_addr == ADDR_WIDTH'(MEM_DEPTH-1))) begin wr_addr <= '0; end else if (wr_en) begin wr_addr <= wr_addr + 1; end end end //============================================ // Write enable //============================================ assign wr_en = wr_valid & (!wr_full); endmodule